MTU Department of Chemical Engineering

2008 Laboratory Safety Inspection Checklist

Room Number:        		Laboratory Supervisor:

Inspector:        	Date:

# Y = Yes     N = No     N/A = Not Applicable

A.  Emergency	Y      N    N/A

- 1.  Completed emergency information labels posted outside room

- 2.  Exits not blocked	         Mandatory

- 3.  Window on laboratory door not covered

- 4.  Emergency Exit route posted on inside of door

- 5.  Fire Extinguisher:	Available within laboratory

- Clearly visible

- Not blocked	         Mandatory

- Annual inspection is up-to-date

- Correct for flammables present

- 6. Safety shower:	Available within laboratory

- Clearly visible

- Not blocked	         Mandatory

- Annual inspection is up-to-date

7.  Eye wash:	Available within laboratory

Clearly visible

Not blocked	         Mandatory

Annual inspection is up-to-date

8.  Spill kits:	Available within laboratory if chemicals present

Clearly visible

Not blocked	         Mandatory

Appropriate for chemicals present (includes mercury)

B.  Housekeeping

- Counters and floor clear of clutter	         Mandatory

- No food or evidence for food

C.  Compressed Gas

- Manual shutoff valves provided at all points of supply and use

- Corrosive gases stored no more than 6-months

- All cylinders secured in place

- Pressure gauges in place

D.  Mechanical Guarding

- Machine guarding in place

2. 	Sharps disposal container available if hypodermic needles present in lab

E.  Electrical

- Electrical cords - not pinched, broken, cracked, or covered up	         Mandatory

- Circuits feeding potentially wet areas protected by GFCI

- No exposed and energized electrical parts

- Electrical panels are not blocked

- Electrical panels labeled, even when not in service

Laboratory Safety Inspection Checklist, p. 2		Lab No.__________

F.  Ventilation	Y      N    N/A

- Laboratory hood sash is in working order - not stuck, broken, or removed

- Equipment or materials do not obstruct air flow

- Hood interior is clean, uncluttered, and not used as storage

- Air flow rate is posted or flow gauge is present(between 80 - 120 fpm)

- Air flow indicator on hood shows positive indication of hood function

G.  Personal Protective Equipment

- Appropriate safety eye wear is worn	         Mandatory

- Appropriate shoes worn	         Mandatory

- Necessary PPE available	         Mandatory

H.  Chemical Storage

- Chemicals in open kept to a minimum	         Mandatory

- Flammable/combustible liquids stored in safety cans or flam. stg. cabinets	         Mandatory

- Flammable storage cabinets grounded

- Incompatible chemicals, i.e. acids and bases, stored separately	         Mandatory

- Chemical containers are properly labeled	         Mandatory

- Chemical storage cabinets labeled

I.  Chemical Waste Management

- Chemical wastes stored in proper containers and location

- All chemical wastes in labeled containers

- All chemical wastes in compatible groups

- Chemical waste disposal done regularly

J.  Hazard Communication

- MSDS sheets available for all non-R&D chemicals in lab	         Mandatory

- Safety Manual accessible

- Emergency response phone numbers permanently marked on the phone

- Chemical inventory is up-to-date and available	         Mandatory

- Standard operating procedures (SOP) are available	         Mandatory

## K.  Glassware

- Properly stored

- Chipped, broken, or scratched glassware is disposed of

- Broken glass container provided

L.  Drying Ovens

1. 	JSA’s for use are on file and followed.

2. 	A logbook is used to document use.

3. 	Direct-heated ovens are not used to dry flammables.

4. 	Mercury thermometers are not used in ovens.

5. 	Ovens used to dry materials that off-gas toxic or noxious vapors are located in a hood.

M.  Compared to last inspection

Improved	  About the same	  Conditions are worse

Laboratory Safety Inspection Checklist, p. 3		Lab No.__________

All items with a NO response are NOT in compliance.

Mandatory items must be in compliance for every inspection – failure to comply might result in immediate laboratory closure.

All other items must be corrected before the next inspection.  These items will become mandatory for the next inspection.

Provide specifics on all items marked “no” above either in the space below or on an attached separate piece of paper.  Also list special hazards or conditions not covered on this form.

Mandatory Items:        In compliance      	Not in Compliance

Signatures of Inspectors: ____________________________________________  Date: ______________

Signature of Lab Supervisor(s): ______________________________________  Date: ______________

Signature of Department Chair: ______________________________________  Date: ______________

Feb, 2008